# ARDF Fox — BaoFeng Fork — Bill of Materials

Callsign: KR8E  |  Hardware: Seeed XIAO ESP32C3 + BaoFeng HT via K1 Speaker-Mic Port  |  Band: 2m VHF  |  v1.0

## Components List

| # | Item / Description | Qty | Specification / Part Number | Source | Est. Cost |
| --- | --- | --- | --- | --- | --- |
| 1 | Seeed XIAO ESP32C3 | 1 | SKU 113991054 — USB-C, built-in LiPo charger | Seeed Studio / Digi-Key | $4.99 |
| 2 | BaoFeng HT (UV-5R or compatible) | 1 | Any K1-port model: UV-5R, UV-5RA, UV-5RE, BF-F8HP, UV-82, UV-82HP, BF-888S | Amazon / HRO / local | $25–45 |
| 3 | BaoFeng K1 speaker-mic cable (donor) | 1 | Cheap $5 speaker-mic — harvest the dual-plug end. Or buy bare K1 plug set. | Amazon / AliExpress | $3–6 |
| 4 | 2N3904 NPN transistor (or BC547) | 1 | TO-92 package; any general-purpose NPN works | Mouser / Digi-Key / Jameco | $0.15 |
| 5 | Resistor 1 kΩ 1/4W | 1 | NPN base resistor (PTT drive current limit) | Any supplier | $0.05 |
| 6 | Resistor 10 kΩ 1/4W | 2 | Audio voltage divider (R1 series, R2 to GND) | Any supplier | $0.10 |
| 7 | Resistor 330 Ω 1/4W | 1 | Status LED current limiter | Any supplier | $0.05 |
| 8 | LED 3mm or 5mm (any color) | 1 | Status indicator | Any supplier | $0.10 |
| 9 | Tactile pushbutton 6mm | 1 | Test / force TX button | Any supplier | $0.20 |
| 10 | 18650 LiPo cell (protected) | 1 | 3.7V 2500–3500 mAh; powers XIAO only — BaoFeng uses its own pack | 18650batterystore.com | $5–8 |
| 11 | 18650 battery holder, 1-cell | 1 | With switch; wire leads to XIAO BAT+/GND pads | Amazon / Mouser | $1–2 |
| 12 | Perfboard / small PCB | 1 | ~5×3 cm for divider + transistor + connectors | Amazon / Digi-Key | $1 |
| 13 | 3.5mm stereo plug (slim-bodied) | 1 | For BaoFeng K1 — use slim insulation; stock plugs may not seat deep enough | Parts Express / Amazon | $1 |
| 14 | 2.5mm stereo plug | 1 | For BaoFeng K1 mic/PTT port | Parts Express / Mouser | $1 |
| 15 | Small enclosure for XIAO board | 1 | ~80×50×20 mm plastic; BaoFeng hangs separately | Hammond / Amazon | $3–5 |
| 16 | Header pins / Dupont wires | 1 | Assorted for XIAO breakout and connections | Amazon | $2 |
| 17 | USB-C cable | 1 | For XIAO programming and charging | On hand | $0 |

## Key Differences vs DRA818V Version

vs DRA818V: No DRA818V module, no LPF required, no UART wiring — far simpler build

vs DRA818V: BaoFeng handles all RF, filtering, and power amplification internally

vs DRA818V: No RF module on perfboard — XIAO + transistor + two resistors is the entire interface

vs DRA818V: Frequency is set manually on the BaoFeng — cannot be changed in firmware

vs DRA818V: BaoFeng runs from its own battery; XIAO from its own 18650

vs DRA818V: Total XIAO-side build cost drops by ~$15 (no DRA818V, no LPF parts)

## Cost Summary

BaoFeng HT (if you don't already own one): +$25–45 vs DRA818V version

XIAO-side build cost: ~$15–20 USD (much simpler — just transistor, resistors, plugs)

If you already own a compatible BaoFeng, this fork is significantly cheaper and faster to build.

NOTE: The UV-5R is the most common compatible radio and is widely available for $25–30. Confirm K1 (2.5mm + 3.5mm dual plug) port before buying.

NOTE: Do NOT use the BF-888S for this — it lacks the 2.5mm/3.5mm speaker-mic port. Use UV-5R series or UV-82 series.

## Battery Life Estimates — XIAO Only

The XIAO side draws very little: ~5 mA average (GPIO logic + sleep). On a 2500 mAh cell:

- Fox 1 (60s TX / 0s sleep): XIAO draws ~10 mA TX + BaoFeng PTT held → ~200 hours XIAO-only runtime

- BaoFeng battery life dominates: UV-5R at 4W draws ~1.8A TX → ~3–4 hours on standard 1800 mAh BL-5

- UV-5R at 1W (Low power): ~600 mA TX → ~10–12 hours on BL-5

- Recommend: set BaoFeng to LOW power (1W) for ARDF — adequate range, much better battery life

NOTE: The BaoFeng's own battery is the limiting factor, not the XIAO's 18650. Carry a spare BL-5 pack for all-day events.

ARDF Fox KR8E (BaoFeng Fork) — BOM v1.0  |  For amateur radio use under FCC Part 97.