# ARDF Fox Transmitter — Bill of Materials

Callsign: KR8E  |  Hardware: Seeed XIAO ESP32C3 + DRA818V  |  Band: 2m (144 MHz)

## Components List

| # | Item / Description | Qty | Specification / Part Number | Source | Est. Cost |
| --- | --- | --- | --- | --- | --- |
| 1 | Seeed XIAO ESP32C3 | 1 | SKU 113991054 — USB-C, built-in LiPo charger | Seeed Studio / Digi-Key | $4.99 |
| 2 | DRA818V VHF transceiver module | 1 | 134–174 MHz, 1W, 3.3V, UART control | eBay / AliExpress | $6–10 |
| 3 | 18650 LiPo cell (protected) | 1 | 3.7V, 2500–3500 mAh, flat-top preferred | 18650batterystore.com | $5–8 |
| 4 | 18650 battery holder (1-cell) | 1 | PCB or wire leads, with switch | Amazon / Mouser | $1–2 |
| 5 | T50-6 powdered-iron toroid | 3 | Amidon T50-6 (yellow), for L1–L3 | Amidon / Mouser | $1/ea |
| 6 | #22 AWG enamelled magnet wire | 1 | ~1 ft; for winding toroids | Local electronics shop | $1 |
| 7 | NP0/C0G capacitor 56 pF | 2 | C0G 50V, 0805 SMD or through-hole (C1, C4) | Mouser / Digi-Key | $0.20 |
| 8 | NP0/C0G capacitor 120 pF | 2 | C0G 50V, 0805 SMD or through-hole (C2, C3) | Mouser / Digi-Key | $0.20 |
| 9 | Resistor 10 kΩ 1/4W | 2 | For MIC audio divider (any tolerance) | Any supplier | $0.10 |
| 10 | Resistor 330 Ω 1/4W | 1 | Status LED current limiter | Any supplier | $0.05 |
| 11 | LED 3mm or 5mm red | 1 | Status indicator | Any supplier | $0.10 |
| 12 | Tactile pushbutton 6mm | 1 | Test/force TX button | Any supplier | $0.20 |
| 13 | SMA female edge-mount connector | 2 | One in (from DRA818V), one out (to antenna) | Mouser / eBay | $1/ea |
| 14 | Perfboard / PCB | 1 | ~5×7 cm, for LPF and interconnects | Amazon / Digi-Key | $1–3 |
| 15 | Coax patch cable SMA–SMA | 1 | ~6 inches, RG316 or RG174 (DRA818V to LPF) | Amazon | $3 |
| 16 | 2m 1/4-wave vertical antenna | 1 | Ground-plane whip or rubber duck 144 MHz | Arrow / HRO | $10–20 |
| 17 | Plastic/aluminium enclosure | 1 | ~100×60×25 mm, for weatherproofing | Hammond / Amazon | $5–10 |
| 18 | Header pins / Dupont wires | 1 | Assorted male/female for XIAO breakout | Amazon | $2 |
| 19 | USB-C cable (for programming) | 1 | For flashing firmware to XIAO | On hand | $0 |

## Cost Summary

Minimum build cost (budget parts): ~$40–50 USD

Typical build cost (quality parts): ~$55–70 USD

NOTE: DRA818V modules vary widely in quality on AliExpress. Order 2 — one for parts.

NOTE: 18650 cells from reputable brands (Sanyo, Samsung, LG) are worth the extra cost for consistent field performance.

## Battery Life Estimates

- 0.5W TX, Fox 1 (60s on / 0s sleep): ~8–10 hours (2500 mAh cell)

- 0.5W TX, Fox 2 (50s on / 10s sleep): ~9–11 hours

- 0.5W TX, Fox 3 (40s on / 20s sleep): ~11–13 hours

- 0.5W TX, Fox 4 (30s on / 30s sleep): ~13–16 hours

- 0.5W TX, Fox 5 (20s on / 40s sleep): ~16–20 hours

- 1W TX, Fox 1 (no sleep): ~4–5 hours

NOTE: ESP32C3 light sleep reduces idle current to ~800 µA. DRA818V in power-down draws ~1 mA. Total off-time current ~2 mA.

NOTE: TX current at 0.5W: ~350 mA total. At 1W: ~600 mA total.

ARDF Fox KR8E — BOM v1.0  |  Prepared for amateur radio use. FCC Part 97 compliant operation required.